## Introduction and Project Background

The City of Albuquerque Parks and Recreation Department, Open Space Division (OSD), with support from the New Mexico State Forestry Division and Ciudad Soil and Water Conservation District, is implementing vegetation management and restoration activities within the Rio Grande bosque on the east side of the river between Campbell Road and Montaño Boulevard. The project is intended to reduce hazardous fuels, improve defensible space, control exotic woody vegetation and noxious weeds, manage accumulated downed woody material, and promote establishment and persistence of resilient, site-adapted native vegetation.

The Middle Rio Grande bosque is a dynamic riparian ecosystem whose vegetation composition and structure have been substantially influenced by river regulation, flood-control infrastructure, altered flooding patterns, wildfire, invasive species, and changing hydrologic conditions. Within the Campbell-to-Montaño project area, these processes have contributed to a heterogeneous vegetation mosaic that includes mature cottonwood forest, dense native and exotic understory vegetation, shrublands, grasslands, disturbed areas, and localized patches of invasive and noxious vegetation. In many locations, dense understory vegetation and accumulated dead and downed woody material create substantial ladder fuels and increase the potential for ground fires to transition into the forest canopy.

GeoSystems Analysis, Inc. (GSA) was retained by OSD to characterize existing vegetation and site conditions and develop spatially explicit recommendations to guide fuels reduction, invasive species management, and post-treatment revegetation. The assessment was designed to support implementation rather than simply document existing vegetation. Accordingly, field observations were integrated with topographic, hydrologic, and remotely sensed information to identify treatment priorities and develop recommendations appropriate to the ecological potential of individual portions of the project area.

The technical approach generally follows the vegetation assessment and restoration planning framework developed by GSA and OSD for the nearby Rio Grande Valley State Park (RGVSP). Using a consistent approach between project areas provides OSD with comparable vegetation information and a repeatable framework for planning fuels reduction, invasive species management, revegetation, and subsequent monitoring throughout the Albuquerque bosque.

## Project Objectives

The primary objective of the assessment was to provide OSD with spatially explicit information needed to plan and implement vegetation management and restoration activities within the Campbell-to-Montaño project area. Specific objectives included:

- Characterize existing riparian vegetation composition and structural conditions using the Hink and Ohmart vegetation classification framework;

- Document native and exotic woody species composition and identify areas with dense understory vegetation, accumulated downed woody material, and other conditions contributing to hazardous fuels;

- Characterize herbaceous vegetation, including grass and forb cover, annual weed abundance, and occurrences of priority invasive and noxious weeds;

- Identify and prioritize areas for fuels reduction, exotic woody vegetation management, annual weed treatment, and noxious weed control;

- Characterize site hydrology and physical setting using field observations, LiDAR-derived relative elevation, available groundwater information, and historic inundation patterns;

- Identify locations appropriate for native revegetation following fuels reduction, including seeding, shrub planting, and cottonwood and tree-willow pole installation;

- Develop site-specific planting densities, species assemblages, and irrigation recommendations that reflect anticipated post-treatment conditions and site hydrologic potential; and

- Provide a spatial framework that can be used by OSD and its partners to guide implementation, prioritize limited resources, and support future monitoring and adaptive management.

## Methods

### Field Assessment and Vegetation Mapping

GSA conducted field reconnaissance and detailed vegetation mapping throughout the project area using methods generally consistent with those applied for vegetation assessment and restoration planning within RGVSP. The project area was divided into relatively homogeneous vegetation map units based on vegetation structure, dominant woody species, understory characteristics, herbaceous cover, disturbance, and other ecologically relevant site conditions.

Vegetation communities were characterized using the Hink and Ohmart riparian vegetation classification framework. Mapping considered both vertical vegetation structure and dominant species composition, allowing differentiation among mature forest, forest with varying understory development, shrub-dominated communities, grasslands, and relatively open or barren areas. Particular attention was given to understory density because of its importance to both habitat conditions and wildfire behavior.

Within each map unit, field crews documented dominant and co-dominant woody species, native and exotic species composition, general canopy and understory structure, grass and forb cover, annual weed cover, and other conditions relevant to vegetation management. Existing native vegetation was specifically considered when developing treatment recommendations so that fuels reduction and exotic species removal could retain desirable native trees and shrubs wherever practicable.

### Exotic and Noxious Vegetation

Exotic woody vegetation was evaluated as both an ecological management concern and a component of hazardous fuel conditions. Particular attention was given to Siberian elm (Ulmus pumila), tree of heaven (Ailanthus altissima), Russian olive (Elaeagnus angustifolia), and saltcedar (Tamarix spp.). Distribution and relative dominance were documented within vegetation map units and subsequently used to identify areas where exotic woody species management should receive greater priority.

Priority noxious and invasive herbaceous species were also mapped during field reconnaissance. Observations were recorded for species warranting targeted management, including Ravenna grass (Tripidium ravennae) and other locally uncommon or potentially expanding invasive species. Annual weed cover was estimated by map unit and considered together with perennial herbaceous cover when evaluating weed-management and revegetation needs.

### Fuels Assessment and Treatment Prioritization

Potential fuels-reduction areas were evaluated using vegetation structure, understory density, exotic woody vegetation cover, and the presence of dead and downed woody material. Dense understory vegetation and downed woody fuels were considered particularly important where they created vertical continuity between surface fuels and the mature cottonwood canopy.

These characteristics were integrated to classify portions of the project area into relative fuels-reduction priority categories. High-priority areas generally represent locations where dense woody understory, exotic vegetation, and accumulated downed material combine to create substantial ladder fuels. Lower-priority areas generally have more open vegetation structure and comparatively limited vertical fuel continuity.

The resulting priority classifications are intended as implementation-planning tools rather than estimates of absolute wildfire risk. They identify locations where vegetation treatment is expected to provide the greatest relative benefit for reducing hazardous fuel structure while supporting OSD's broader ecological restoration objectives.

Topography, Hydrology, and Site Potential

Physical and hydrologic conditions were evaluated to help determine the ecological potential of individual portions of the project area and guide revegetation recommendations. A high-resolution relative elevation model was developed using 2022 LiDAR data to characterize floodplain topography and identify relatively low-lying and elevated surfaces. Available groundwater information was reviewed in conjunction with relative elevation to evaluate likely differences in depth to groundwater across the site.

Historic floodplain inundation was evaluated using available high-resolution imagery acquired during Rio Grande runoff conditions at multiple river discharges. These observations were used to identify portions of the project area that are comparatively more likely to experience surface-water inundation and areas that are largely disconnected from contemporary flooding.

Field soil observations supplemented the desktop assessment. Collectively, relative elevation, groundwater conditions, soil characteristics, inundation potential, and existing vegetation provided the basis for evaluating site hydrologic potential and selecting appropriate revegetation strategies.

### Development of Management and Revegetation Recommendations

Management recommendations were developed by integrating existing vegetation conditions with physical and hydrologic site characteristics. Rather than applying a uniform treatment prescription across the project area, the analysis was designed to recognize differences in existing vegetation, treatment need, and ecological potential.

Spatial priority classes were developed for fuels reduction, annual weed management, and management of selected exotic woody species. Revegetation recommendations were similarly developed for herbaceous seeding, native shrub planting, and cottonwood and tree-willow pole installation. Recommended shrub assemblages and planting densities considered relative elevation, inundation potential, canopy closure, existing understory composition, and anticipated vegetation conditions following fuels reduction. Irrigation intensity was varied according to anticipated establishment requirements and site hydrologic conditions.

Tree pole planting recommendations considered existing overstory composition, soil conditions, and predicted groundwater accessibility. Areas expected to retain adequate native overstory following fuels treatments generally received lower planting priority, while locations where removal of exotic woody vegetation could substantially reduce overstory cover were assigned greater priority.

The resulting maps and prescriptions provide a conceptual restoration framework rather than a requirement that all recommended treatments occur simultaneously. Implementation can be phased based on funding, treatment schedules, site access, vegetation response, and OSD management priorities. This approach also allows treatment outcomes to be evaluated over time and subsequent management or revegetation activities to be adjusted through adaptive management.

## Results

Inundation Mapping

Surface inundation into this unit during spring runoff flows is relatively minor, especially at discharges below 4000 cfs (as measured at the USGS “Rio Grande at Albuquerque” gage near Central Bridge). At these low discharges, what little inundation that occurs is restricted exclusively to lower elevation portions of the bank line. At discharges above 4000 cfs, inundation increases through the unit, though still remains modest. Inundation at these higher discharges is mostly present in the previously constructed flow-through channel that passes through the unit.

Table 1. Acreage of Inundation within the Montaño unit based on satellite image analysis from 2019.

|  |  | Discharge (cfs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | Parameter | 1630 | 1970 | 2560 | 4240 | 4820 | 5020 |
| Montaño | Inundation (acres) | <1 | <1 | <1 | 1.7 | 3.5 | 8.1 |
| Montaño | Inundation (%) | <1% | <1% | <1% | 1.4% | 2.7% | 6.4% |

The minimal inundation experienced at this location is in large part due to the average elevation above the river surface (4.9 ft above the river surface at ~600 cfs for the entire unit).

Summary of Elevation Trends

There are some slight depressional features in the land surface, however most of the area lies within 3-5 ft above the water surface. The most significant depressional feature in the unit is the flow through channel constructed by the U.S. Army Corps of Engineers in 2008. Besides this channel, and some adjacent depressions likely formed by relict channels, the unit is primarily an elevated floodplain.

General Vegetation Types

This unit is characterized by a mosaic of different forest types, most of which contain substantial understory canopy. There are some dense mixed riparian stands along the bank line, however most contain upper-level forest canopy as well. Additionally, there is a small portion of shrubland immediately adjacent to the bike path atop the levee bordering the unit’s east side. Grasslands are restricted to two large, adjacent units in the north-central portion of the unit, while open areas are minimal. Map A1 shows the distribution of general vegetation types.

Table 2. Acreage of general vegetation types in the Montaño unit.

| Unit | Dense, tall shrubs | Forests with little to no understory | Forests with relatively dense understory | Grassland | Open/barren | Short shrubs | Grand Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Montaño | 1.2 | 6.2 | 108.6 | 7.7 | 0.5 | 2.5 | 126.7 |

Forest structure generally gets denser, particularly in the understory, approaching the Rio Grande. Additionally, most of the area near the river also displays significant buildup of dead and downed trees in the understory. These downed snags, combined with a dense understory, present significant ladder fuel risk.

Woody Species Dominance

Rio Grande Cottonwood (Populus deltoides ssp. wislezeni) is the most common woody species observed in the overstory throughout the unit, being mapped in nearly every portion of the area. Further, cottonwood is a dominant or co-dominant overstory species in the majority of the mapped forest types. Notably, many cottonwoods showed some signs of canopy dieback throughout the unit, particularly in areas further away from the river. Siberian Elm (Ulmus pumila) is also a common component throughout the unit, being present in well over half of the mapped forest types. However, Siberian Elm is not as often a dominant or co-dominant species in the overstory when compared to cottonwood, suggesting a lower overall density.

The understory is dominated largely by Russian Olive (Elaeagnus angustifolia) and Saltcedar (Tamarix spp.) intermixed with native species like New Mexico Olive (Forestiera Neomexicana), Coyote Willow (Salix exigua), and others. Both Russian Olive and Saltcedar are present in nearly every mapped forest type, and are dominant or co-dominant in nearly half of all mapped areas. Both species aren’t present in monotypic stands, however their density increases near the river and both species contribute to the dense thicket of ladder fuels present near the channel. Tree of Heaven (Ailanthus altissima) is widespread through the unit, however in most instances it occurs in small, isolated patches. The density does increase in the northern section of the unit however, where it forms a co-dominant component of the understory in one mapped area.

While not co-dominant in any of the mapped areas, native species like New Mexico Olive, False Indigobush (Amorpha fruticosa), and Seep Willow (Baccharis salicifolia) are widespread throughout the entire mapped area. The density and vigor of these populations appeared generally healthy, indicating that they are suitable choices for post-clearing revegetation efforts. The short shrubby areas are dominated exclusively by the native Fourwing Saltbush (Atriplex canescens) and Sand Sagebrush (Artemisia filifolia). See map A2 for detailed distribution of Hink and Ohmart vegetation types.

Annual Weeds

The dominant annual weeds in the unit are Kochia (Bassia scoparia) and tumbleweed (Salsola tragus) which have modest but variable cover throughout the area. Also widely distributed is the non-native sweetclover (Melilotus officinalis), though this species generally has lower density than kochia or tumbleweed. Overall annual weed cover in the unit is in the lower density classes (10% cover and below) through more than half the mapped area. Locations with higher weed cover are generally more patchy and isolated, with the highest densities occurring in the most highly disturbed (through foot traffic and previous wildfire) portions of the unit. Map A3 shows the distribution of annual weed cover by map unit.

Table 3. Acreage of annual weed cover class in the Montaño unit.

| Unit | <5% | 6-10% | 11-25% | 26-50% | 51-75% | 76-90% | Grand Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Montaño | 32.5 | 45.0 | 45.0 | 3.7 | 0.5 | 0 | 126.7 |

Maintenance Recommendations

Priority fuels treatment locations

Fuels reduction treatment locations were prioritized based on a variety of site-specific variables, principally understory canopy density, high cover of exotic woody species, and the presence of ample downed wood snags that form ladder fuels. Exactly half of the unit’s acreage lies within the high priority class, with most of this area lying in the portions of the area closer to the river. In these areas, the understory canopy is incredibly dense and dominated by exotic species. Coupled with significant buildup of downed snags and modest annual weed cover, the potential for ground fires to evolve into canopy fires is very high. The lowest priority areas in this unit are those that have an open structure, with little to no presence of ladder fuels. Map A4 includes the distribution of different fuels treatment priority classes.

Table 4. Acreage of fuels reduction priority class in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 63.3 | 48.4 | 15.0 | 126.7 |

Siberian Elm

Current cover and dominance of Siberian elm was used to prioritize management of this species within individual segments of each site (Table 5). Siberian elm is widespread, but with modest cover throughout most of the unit. The vast majority of the mapped acreage has moderate need for management of Siberian elm. Where the species presents the biggest problem is in the portions of the unit with the thickest understory, where Siberian elm acts in conjunction with other exotic species, plus significant quantities of downed wood, to form a large mass of ladder fuels. Immediate treatment in these zones should focus on a combination of mechanical removal and systemic herbicide application to prevent resprouting and reduce seed dispersal. Seed dispersal reduction is especially important while germination safe locations are abundant in the sites following treatment. Map A5 contains Siberian elm priority classes.

Table 5. Acreage of Siberian elm priority classes in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 7.1 | 88.1 | 31.5 | 126.7 |

Tree of Heaven

Tree of heaven priority classes were developed using an identical framework to the one developed for Siberian elm (Table 6). Though widespread throughout the unit, Tree of Heaven is most often growing in small, isolated patches. Larger populations occur in the northern portion of the unit near the river. Targeted herbicide application paired with revegetation using competitive native species is recommended. Seed dispersal reduction is especially important while germination safe locations are abundant in the sites following treatment. Map A6 shows the distribution of tree of heaven treatment priority classes.

Table 6. Acreage of tree of heaven priority class in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 1.6 | 92.7 | 32.4 | 126.7 |

Annual Weeds

A combination of annual weed, perennial grass, and perennial forb cover was used to prioritize annual weed management by map unit (Table 7). Well over half of the unit lies in the low treatment priority class, and only a small area falls into the highest priority. Long term control of annual weeds will be complicated by regular foot traffic along trails through the area. High and moderate priority treatment areas should be a top focus for early intervention, particularly where annual weeds co-occur with ladder fuels. Incorporating aggressive chemical control techniques and post-treatment monitoring to address root suckering and seedling emergence is required for long term control. Map A7 contains the annual weed priority class distribution.

Table 7. Acreage of annual weed priority class in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 4.3 | 45.0 | 77.4 | 126.7 |

Noxious Weeds

Ravenna grass (Tripidium ravennae) is the only listed noxious weed found in the unit, with several populations occurring throughout. Most populations are isolated and small, however significant populations occur along sandbars just outside the unit. This nearby seed source will likely contribute to continued population growth if left unaddressed. Other species observed are not currently listed as noxious weeds in New Mexico, but are potentially aggressive enough to warrant treatment. Yellow bluestem (Bothriochloa ischaemum) is a watch listed species that aggressively spreads in highly disturbed areas (such as the trailside environment it was observed in here). Left unaddressed, this species will readily spread through the area. Myrtle spurge (Euphorbia myrsinites) was also observed and, while not considered a weed in New Mexico, is unusual to see in the bosque. It is a watch listed species in Colorado and readily spreads in arid environments. Spread of this species will likely be low to modest, but its unusual presence and small population does provide an ideal opportunity for early control. Eradication of these species is feasible with repeated mechanical and chemical control and on-going monitoring. Map A8 shows noxious weed observations within the project area.

Table 8. Number of noxious weed observations in the Montaño unit.

| Unit | Ravenna grass | Yellow bluestem* | Myrtle spurge** | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 10 | 1 | 1 | 12 |

* Yellow bluestem (Bothriochloa ischaemum) is a watch-listed species in New Mexico.

** Myrtle spurge (Euphorbia myrsinites) is not classified as noxious in New Mexico but is a novel exotic species in the bosque.

Revegetation Recommendations

Seeding

A variety of site conditions were used to prioritize specific locations for seeding within this unit. Seeding is ideally performed soon after fuels reduction treatments in order to compete with resprouting weedy species. The majority of the unit lies in the low priority class, however moderate need does exist in some parts of the mapped area. Seeding in these units can reinforce restoration efforts, suppress undesirable vegetation, and improve species richness across structural layers once fuels management has been completed. Map A9 contains seeding priority zones. A recommended seed mix is included at the end of this memo (Table 16).

Table 9. Acreage of seeding priority zones in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 4.9 | 45.9 | 75.9 | 126.7 |

Shrub planting density

Shrub planting density was prioritized within individual portions of each site using a combination of environmental conditions, with the goal of increasing native understory cover and diversity once fuels management has been completed. Most of the area lies in the high-density class, due to relatively sparse understory expected after fuels treatments. Much of the unit currently exhibits forest structures with dense understory, however the principal components of this understory are exotic woody species. Revegetation with native shrubs can help boost ecological function and limit the spread of exotic trees that eventually form ladder fuels. Map A10 contains the shrub planting density classes.

Table 10. Acreage of shrub density classes in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 72.9 | 18.7 | 35.1 | 126.7 |

Shrub mix recommendations for revegetation were developed using a suite of site-specific physical and ecological variables, including relative elevation (based on a high-resolution elevation model), inundation frequency, overstory canopy closure, and current understory composition. Map A11 shows the recommended shrub mix while Map A12 shows the shrub planting priority areas.

Table 11. Acreage of recommended shrub planting mix in the Montaño unit.

| Unit | All Riparian Shrubs | All Xeroriparian Shrubs | Even Mix Riparian and Xeroriparian Shrubs | Mostly Riparian Shrubs | Mostly Xeroriparian Shrubs | Grand Total |
| --- | --- | --- | --- | --- | --- | --- |
| Montaño | 6.7 | 1.8 | 33.7 | 75.8 | 8.7 | 126.7 |

Shrub irrigation intensity

Shrub irrigation intensity recommendations across the project area were guided by a combination of hydrologic and ecological variables, including canopy closure and relative elevation above the river. Map A13 contains the shrub irrigation intensity.

Table 12. Acreage of shrub irrigation intensity class in the Montaño unit.

| Unit | High | Moderate | Low | None | Grand Total |
| --- | --- | --- | --- | --- | --- |
| Montaño | 1.7 | 45.3 | 79.0 | 0.7 | 126.7 |

Tree pole planting

Map units were prioritized for tree pole planting based on existing vegetation type and predicted depth to groundwater. A majority of the project area is considered low priority for pole planting, largely because there is anticipated to be sufficient overstory cover after fuels reduction of exotic woody species. Some moderate priority areas are anticipated however, particularly in areas where the overstory contains a significant portion of exotic species. Map A14 shows the tree pole installation priority class distribution.

Table 13. Tree pole installation priority class in the Montaño unit.

| Unit | High | Moderate | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 2.5 | 47.7 | 76.5 | 126.7 |

Pole density

Similar to pole planting priority, pole density estimates are based on existing vegetation type and structure, coupled with anticipated needs after fuels reduction has been completed. As Rio Grande Cottonwood is the most common overstory species, and a species targeted for protection, overall pole density needs are estimated to be low. Higher densities should be prioritized in the small areas with exotic woody species heavily present in the overstory canopy. Map A15 shows the recommended tree pole density class distribution.

Table 14. Tree pole density class acreage in the Montaño unit.

| Unit | High | Medium | Low | Grand Total |
| --- | --- | --- | --- | --- |
| Montaño | 0.6 | 3.3 | 122.8 | 126.7 |

Pole species mix

Table 15 summarizes the mix of tree pole species for installation across project units, distinguishing areas prioritized for tree willow (TW), cottonwood (C), or combinations of the two. These recommendations were developed based on predicted depth to groundwater, soil texture, and the existing species composition, coupled with expected composition post fuels reduction. Map A16 contains the recommended tree pole installation mix.

Table 15. Acreage of the tree pole installation mix in the Montaño unit.

| Unit | All Tree Willow | All Cottonwood | Even Mix Cottonwood and Tree Willow | Mostly Tree Willow | Mostly Cottonwood | Grand Total |
| --- | --- | --- | --- | --- | --- | --- |
| Montaño | 3.9 | 1.8 | 44.9 | 67.4 | 8.7 | 126.7 |

## Recommendations and Adaptive Management

Hydrologic and vegetation conditions observed in 2026 indicate that restoration and vegetation management within the Campbell-to-Montaño project area should increasingly emphasize resilience to changing hydrologic conditions rather than attempting to maintain a uniform historical cottonwood-dominated bosque throughout the project area. Much of the site is an elevated floodplain with limited contemporary inundation, and cottonwood canopy dieback was observed during field assessments, particularly farther from the river. Due to prolonged channel drying, above average temperatures, and low precipitiation, much of the cottonwood canopy has senesced leaves by late August 2026.

The following recommendations are intended to guide implementation and provide a framework for adaptive management:

- Recognize 2026 hydrologic conditions as an important change in the restoration context.

  - Hydrologic conditions observed along the Rio Grande through Albuquerque during 2026 are substantially drier than conditions under which much of the existing mature bosque developed.

  - The project area is already relatively disconnected from contemporary overbank flooding. Most of the site lies approximately 3 to 5 feet above the river during baseflow conditions, and inundation remains limited even during relatively high Rio Grande discharges.

  - Continued groundwater decline, infrequent overbank flooding, and prolonged periods of river drying may increasingly constrain the ability to maintain cottonwood- and willow-dominated and riparian vegetation on elevated floodplain surfaces.

  - Its becoming increasingly clear that restoration prescriptions should therefore be based on the current and anticipated hydrologic potential of individual sites, rather than assuming that historical vegetation communities can or should be maintained everywhere.

- Manage toward a heterogeneous and resilient bosque rather than a uniform vegetation condition.

  - Retain and protect healthy native cottonwoods and native understory vegetation wherever practicable.

  - Continue to favor traditional riparian vegetation in lower-elevation areas where groundwater and inundation conditions remain suitable.

  - On higher terraces and hydrologically disconnected surfaces, increasingly emphasize native xeric and xeroriparian vegetation capable of persisting with deeper groundwater and limited flooding.

  - Maintain a mosaic of mature forest, open woodland, native shrubland, and perennial herbaceous communities rather than attempting to establish continuous woody canopy.

- Establish a resilient native perennial herbaceous community immediately following disturbance.

  - Treat native seeding as an integral component of fuels reduction and exotic woody vegetation removal rather than as an optional follow-up activity.

  - Seed disturbed areas as soon as practicable following vegetation removal to occupy newly available growing space before annual weeds and exotic woody seedlings become established.

  - Favor diverse mixtures of native perennial grasses and forbs adapted to the hydrologic and soil conditions of individual treatment areas.

  - Include both warm- and cool-season species where appropriate to increase seasonal resource availability and ecological resilience.

  - Establishing perennial herbaceous cover should serve multiple objectives, including soil stabilization, weed suppression, habitat improvement, increased native species richness, and reduction of exposed disturbed ground.

  - Focus on establishing flowering species with high pollinator value.

- Make annual weed management a high priority following fuels reduction.

  - Recognize that annual weed conditions mapped prior to treatment may not accurately represent conditions following substantial vegetation removal.

  - Be prepared to aggressively manage annual weeds if there is widespread cottonwood dieback. Rapid removal of dense woody canopy and understory vegetation can substantially increase light availability and disturbance at the soil surface, creating favorable conditions for annual weeds.

  - Kochia and tumbleweed are already widespread in portions of the project area, and the highest existing annual weed densities occur in disturbed areas.

  - Reevaluate annual weed management priorities following fuels treatments rather than relying exclusively on pretreatment priority maps.

  - Monitor treated areas during the first growing season following vegetation removal, when early intervention is most likely to prevent large weed populations from becoming established.

  - Prioritize treatment before annual weeds mature and contribute additional seed to the soil seed bank.

  - Coordinate weed treatment and native seeding so that control of undesirable vegetation is followed by establishment of competitive perennial native vegetation.

  - Continue monitoring for multiple years because disturbance effects and seed-bank responses may persist well beyond the initial treatment year.

  - Increase treatment intensity where repeated monitoring indicates persistent annual weed dominance.

- Initially emphasize xeric and xeroriparian woody species on elevated terraces.

  - Prioritize drought-tolerant native shrubs and small trees where groundwater conditions may no longer reliably support cottonwood or willow.

  - Existing native species performing well within the project area—including New Mexico olive, fourwing saltbush, and sand sagebrush—provide an important foundation for this approach.

  - Expand the planting palette to include additional native woody species adapted to xeric floodplains, arroyos, desert washes, and lower-elevation riparian systems of New Mexico and the Southwest.

  - Potential species for evaluation include netleaf hackberry, desert willow, screwbean mesquite, honey mesquite, New Mexico olive, skunkbush sumac, pale wolfberry, Apache plume, fourwing saltbush, desert hackberry, and other drought-adapted native species.

  - Species that occur outside their current or historical prevalence in the Albuquerque bosque should initially be considered experimental climate-adaptation species rather than broadly prescribed restoration species.

  - Retain existing cottonwoods while establishing drought-tolerant woody vegetation beneath and between them so that future loss of mature cottonwoods results in a gradual transition in vegetation structure rather than abrupt loss of woody habitat.

- Treat initial xeric woody plantings as a pilot study.

  - Avoid committing immediately to a single xeric planting palette across large portions of the bosque.

  - Establish multiple species and species combinations across representative elevations, soil conditions, groundwater depths, canopy conditions, and treatment types.

  - Where practical, replicate plantings sufficiently to distinguish species performance from localized site effects.

  - Track survival, vigor, growth, canopy development, drought stress, herbivory, recruitment, irrigation requirements, and other indicators of establishment success.

  - Compare plant performance with groundwater depth, relative elevation, soil characteristics, existing canopy cover, and irrigation history.

  - Use monitoring results to progressively refine species palettes, planting densities, installation methods, and irrigation prescriptions.

  - Expand successful treatments in subsequent phases and discontinue or modify approaches that consistently perform poorly.

  - This approach is consistent with the report's existing recommendation that treatments be phased and adjusted over time based on vegetation response and adaptive management.

- Continue aggressive early detection and rapid response for noxious and emerging invasive species.

  - Continue targeted monitoring and treatment of Ravenna grass.

  - Account for nearby Ravenna grass populations outside the project boundary that may provide continuing seed sources.

  - Continue surveillance for yellow bluestem, which has demonstrated an ability to spread aggressively in disturbed environments.

  - Treat small populations of novel exotic species while eradication remains practical.

  - The existing assessment identified Ravenna grass, yellow bluestem, and myrtle spurge as species warranting continued attention and concluded that repeated treatment and monitoring could potentially eradicate or contain existing populations.

- Plan for multiple years of follow-up treatment of exotic woody species.

  - Treat removal of Siberian elm, tree of heaven, Russian olive, and saltcedar as the beginning of a management sequence rather than completion of treatment.

  - Monitor for root suckering, stump resprouting, seedlings, and new recruitment following initial clearing.

  - Conduct follow-up herbicide treatment where necessary.

  - Prioritize control of seedlings while plants are small and treatment costs remain relatively low.

  - Rapidly revegetate areas where exotic woody removal substantially reduces understory or overstory cover.

  - Establish competitive native woody and herbaceous vegetation to reduce opportunities for exotic species to recolonize disturbed areas.

- Monitor groundwater conditions and explicitly incorporate groundwater into future restoration planning.

  - Establish or continue long-term groundwater monitoring within and adjacent to the project area.

  - Where possible, use continuous pressure transducers rather than relying solely on occasional manual groundwater measurements.

  - Characterize seasonal groundwater fluctuations and annual groundwater minima, not simply average groundwater elevation.

  - Evaluate groundwater conditions alongside cottonwood canopy vigor, shrub survival, and revegetation performance.

  - Use groundwater data to distinguish areas where traditional cottonwood-willow restoration remains viable from areas where xeric or xeroriparian vegetation represents a more sustainable long-term strategy.

  - Over time, develop empirical relationships between depth to groundwater and vegetation response that can guide future planting decisions throughout RGVSP and the broader Albuquerque bosque.

  - Reevaluate tree-pole planting prescriptions if monitored groundwater conditions differ substantially from the conditions assumed during development of this assessment. Tree-pole recommendations in the current plan are explicitly based in part on predicted groundwater accessibility.

- Continue cottonwood and willow restoration where hydrologically appropriate, but be increasingly selective.

  - Cottonwood and tree-willow pole installation should be concentrated in locations where groundwater monitoring or other site information indicates a reasonable likelihood of long-term survival.

  - Avoid investing heavily in riparian pole plantings on elevated terraces where groundwater conditions are unlikely to support mature trees without prolonged supplemental irrigation.

  - Retain existing mature cottonwoods wherever they are healthy and compatible with fuels-management objectives.

  - Use observed canopy condition as an additional indicator of site suitability; widespread stress or mortality of existing mature cottonwoods may indicate poor prospects for establishing new cottonwood without changes in hydrologic conditions.

- Integrate vegetation monitoring directly into implementation.

  - Establish baseline conditions prior to treatment where practical.

  - Monitor native woody planting survival and vigor.

  - Monitor perennial grass and forb establishment.

  - Monitor annual weed cover and composition.

  - Monitor noxious and emerging invasive species.

  - Monitor resprouting and recruitment of exotic woody vegetation.

  - Monitor cottonwood canopy condition, including dieback and mortality.

  - Monitor groundwater elevation and seasonal groundwater minima.

  - Monitor changes in vegetation structure and fuel continuity following treatment.

  - Establish permanent photo points at representative treatment areas.

  - Consider repeat high-resolution aerial or drone imagery to quantify changes in canopy cover, vegetation structure, treatment footprint, and vegetation recovery.

  - Repeat selected field vegetation measurements to provide quantitative measures of restoration trajectory.

  - Maintain monitoring locations across the site's hydrologic gradient so that vegetation responses can be interpreted relative to environmental conditions.

- Use monitoring results to drive adaptive management.

  - Periodically review vegetation, groundwater, weed, and fuels-monitoring results together rather than evaluating each independently.

  - Modify woody species palettes based on observed survival and performance.

  - Modify seeding mixtures where establishment is poor or annual weeds remain dominant.

  - Adjust planting densities based on observed canopy development and fuel structure.

  - Reevaluate sites where plantings require persistent supplemental irrigation, as those species may not represent sustainable long-term restoration choices.

  - Adjust annual weed and exotic species treatment priorities in response to post-treatment vegetation conditions.

  - Use successful pilot treatments as templates for subsequent restoration phases elsewhere within RGVSP.

  - Document unsuccessful treatments as well as successful ones so that future projects do not repeatedly invest in approaches poorly suited to current hydrologic conditions.

- Use the project as an opportunity to develop a broader climate-resilient restoration strategy for the Albuquerque bosque.

  - The Campbell-to-Montaño project can provide an important demonstration of how bosque restoration strategies may need to evolve as hydrologic conditions change.

  - The long-term objective should be a diverse, native, structurally heterogeneous vegetation mosaic capable of persisting across a range of hydrologic conditions.

  - Successful management should be measured not solely by maintaining historical vegetation composition, but by maintaining native vegetation cover, habitat structure, plant diversity, wildlife value, soil stability, invasive-species resistance, and acceptable fuel conditions.

  - Active establishment of xeric woody species and resilient perennial herbaceous vegetation before widespread cottonwood mortality occurs provides an opportunity to guide ecological transition rather than respond to it after substantial canopy loss.

  - This framework should remain flexible as additional information becomes available regarding groundwater conditions, cottonwood response, revegetation success, and future Rio Grande hydrology.

Table 16. Recommended seed mix

|  |  | Rate (pls lbs/acre) |  |  |
| --- | --- | --- | --- | --- |
| Common Name | Species Name | General Lifeform | Upland | Bosque |
| Indian ricegrass | Achnatherum hymenoides | Graminoid | 2.5 | 2.5 |
| Sand sage | Artemisia frigida | Shrub | 0.5 | 0.0 |
| Four-wing saltbush | Atriplex canescens | Shrub | 0.3 | 0.0 |
| Horsetail milkweed | Asclepias subverticillata | Forb | 1.0 | 1.0 |
| Side oats gramma | Bouteloua curtipendula | Graminoid | 1.0 | 1.0 |
| Blue gramma | Bouteloua gracilis | Graminoid | 0.5 | 0.0 |
| Rocky mountain bee plant | Cleome serrulata | Forb | 0.5 | 0.5 |
| Inland saltgrass | Distichlis spicata | Graminoid | 0.0 | 2.0 |
| Canada wildrye | Elymus canadensis | Graminoid | 1.0 | 1.0 |
| Bottlebrush squirreltail | Elymus elymoides | Graminoid | 2.0 | 2.0 |
| Slender wheatgrass | Elymus trachycaulus | Graminoid | 1.0 | 1.0 |
| Annual sunflower | Helianthus annuus | Forb | 3.0 | 3.0 |
| Hairy goldenaster | Heterotheca vilosa | Forb | 0.5 | 0.5 |
| Green sprangletop | Leptochloa dubia | Graminoid | 0.5 | 1.0 |
| Hoary tansyaster | Machaeranthera cansecens | Forb | 0.5 | 0.5 |
| Whitestem blazing star | Mentzelia albicaulis | Forb | 0.5 | 0.5 |
| Wild four oclock | Mirabilis multiflora | Forb | 0.5 | 0.5 |
| Hooker evening primrose | Oenothera hookeri | Forb | 0.5 | 0.5 |
| Pale evening primrose | Oenothera pallida | Forb | 0.5 | 0.5 |
| Vine mesquite | Panicum obtusum | Graminoid | 1.5 | 1.5 |
| Western wheatgrass | Pascopyrum smithii | Graminoid | 3.0 | 3.0 |
| Galleta | Pleuraphis jamesii | Graminoid | 1.5 | 0.0 |
| Gray headed prairie coneflower | Ratibida pinnata | Forb | 0.5 | 0.5 |
| Scarlet globemallow | Sphaeralcea coccinea | Forb | 0.5 | 0.5 |
| Copper globemallow | Sphaeralcea angustifolia | Forb | 0.5 | 0.5 |
| Gooseberry leaf globemallow | Sphaeralcea grossularifolia | Forb | 0.5 | 0.5 |
| Alkali sacaton | Sporobolus airoides | Graminoid | 1.0 | 1.5 |
| Spike sacaton | Sporobolus contractus | Graminoid | 0.3 | 0.3 |
| Sand dropseed | Sporobolus cryptandrus | Graminoid | 0.3 | 0.3 |
| Mesa dropseed | Sporobolus flexuosus | Graminoid | 0.3 | 0.0 |
| Giant dropseed | Sporobolus giganticus | Graminoid | 0.5 | 0.0 |
| Giant sacaton | Sporobolus wrightii | Graminoid | 1.0 | 1.5 |
| TOTAL |  |  | 28.2 | 28.1 |